# USB Communication

The Spectran uses byte parallel transport via USB. The communication can be established via the FTD2XX library form Future Technology Devices International Ltd.

To avoid misuse, the PC has to identify itself by the verify command. Because the state of the USB communication may be unknown, it’s recommended to try the verify command at least twice.

All USB Constants are defined in the file usbcmd.h

Each command start with the command id followed by some parameters. The Spectran responds with a suitable status if the command is accepted. Three commands differ from that: There is no response to wrong VERIFY commands, there is no response to the LOGOUT command and the AMPFREQDAT, which carries current measurement data, is sent without request.

Word (2 Bytes) values are transfered in lowbyte first, highbyte last, which matches the normal Intel Little Endian order.

## VERIFY

This is a sequence of 5 bytes, starting with the command UCMD_IDREQ, followed by UIDREQ_CODE1 - UIDREQ_CODE4. The expected answer is 5 bytes starting with UST_IDACK followed by UIDACK_CODE1 - UIDACK_CODE4

Send:

| 0x01 | 0xA5 | 0x5A | 0xF1 | 0x1F |
| --- | --- | --- | --- | --- |

Receive:

| 0x01 | 0x51 | 0x1A | 0xF5 | 0xAF |
| --- | --- | --- | --- | --- |

If the send command doesn’t match, nothing is sent back and the communication timeouts.

## LOGOUT

Send this sequence to resets the verified state in the spectran. The Spectran won’t accept any commands but VERIFY. Therefore it won’t send back any data to a custom application which accidently communicates with it.

Send:

0x02

Receive:

Nothing

## FILEREAD

The UCMD_FILEREAD command reads a file block. The command is followed by 2 bytes fileid and 2 bytes offset. The offset is in words, not bytes. The Command responds with UST_FILESTAT followed by a status byte. If the status byte is USFT_DONE, the command is followed by UST_FTREADDAT, 1 byte data lenght in words (not bytes) and data.

Send:

| 0x10 | id | id | offset | offset |
| --- | --- | --- | --- | --- |

Receive:

| 0x10 | status |
| --- | --- |

Receive

| 0x12 | Len | data | .... |
| --- | --- | --- | --- |

The status value UFST_DONE indicates successfull execution of the request, read requests are followed by a UST_FTREADDAT command response. The status value UFST_NOTFND indicates that the file doesn’t exist, the status value UFST_FSFULL indicates full file system (for write commands only), the status command UFST_ACCERR indicates a general Spectran internal error.

## FILEWRITE

UCMD_FILEWRITE appends a block of data to a file. For an update, the file must be deleted first and then completely written. The length of the command must not exceed the maximum buffer size as returned by FTMAXSIZE, currently 256 bytes.

Send:

| 0x11 | id | id | len | data | .... |
| --- | --- | --- | --- | --- | --- |

Receive:

| 0x10 | status |
| --- | --- |

Status indicates write success or error as described in FILEREAD.

## FTMAXSIZE

Use this command to query the maximum block size for one transfer command. The returned value indicates the buffer size in words, not bytes.

Send:

0x13

Receive:

| 0x11 | Len |
| --- | --- |

## FILEDEL

UCMD_FILEDEL deletes a file.

Send:

| 0x12 | id | id |
| --- | --- | --- |

Receive:

| 0x10 | Status |
| --- | --- |

Status indicates delete success or error as described in FILEREAD.

## GETSTPVAR

UCMD_GETSTPVAR reads a Spectran environment variable. The value is casted as 4 byte float variable in little endian format (intel x86).

Send:

| 0x20 | Id | Id |
| --- | --- | --- |

Receive:

| 0x20 | Status | val | val | val | val |
| --- | --- | --- | --- | --- | --- |

## SETSTPVAR

UCMD_SETSTPVAR sets a Spectran environment variable. The value is casted as 4 byte float variable in little endian format (intel x86).

Send:

| 0x21 | id | id | val | val | val | val |
| --- | --- | --- | --- | --- | --- | --- |

Receive:

| 0x21 | Status |
| --- | --- |

Status indicates success by USST_DONE and failure by USST_FAILED.

## AMPFREQDAT

UST_AMPFREQDAT is somewhat special because it has no request. It’s sent by Spectran as long as the environment Variable USBMEAS is set. The data is total 16 bytes separated as follows:

| Time | unsigened int (4 bytes) The internal time in milliseconds since start. |
| --- | --- |
| Frequency | unsigened int (4 bytes) The frequency in Hz / 10. a Value auf 1000 means 10 kHz |
| Minval | Float (4 bytes) The minimum value in dBm measured for the current point |
| Maxval | Float (4 bytes) The maximum value in dBm measured for the current point |

Send:

Nothing

Receive:

| 0x22 | time | time | time | time | freq | freq | freq | freq |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| min | min | min | min | max | max | max | max |  |  |

Explanation of Minval, Maxval and Pulsemode:

Each measurement point is captured multiple times before it's displayed. If the Pulsemode setting is on, and the maximum value during the time is high enough, then the value is reported even if the signal isn't detected during the whole period, so minval is low or -120 dBm. If the Pulsemode setting is off, the value is suppressed. As long as the input is continuos, it's better to leave the setting off. If the signal changes very fast (time slot signals), it's recommended to turn the setting on.

## Other commands

The Spectran flash update is done also via USB command. The start command is 0xE0, which activates a special flash mode. The detailed Flash procedure is outside the scope of this document.

# Spectran Environment Variables

The complete operation of the Spectran is controlled by Environment variables. This includes the sweep range, sweep mode and the sweep results.

| ID | ## Name | Description |
| --- | --- | --- |
| 0x01 | STARTFREQ | Start frequency of the sweep in MHz. |
| 0x02 | STOPFREQ | Stop frequency of the sweep in MHz. If the Span is 0, the display changes from frequency domain to time domain. |
| 0x03 | RESBANDW | Resolution bandwidth. 0: Full 1: 3 MHz 2: 1 MHz 3: 300 kHz 4: 100 kHz |
| 0x04 | VIDBANDW | Video bandwidth, the values are the same as RESBANDW. |
| 0x05 | SWEEPTIME | Sweep time in milliseconds. |
| 0x06 | ATTENFAC | Attenuation factor, -10 means automatic. Range if from 0 dBm to 30 dBm. The Value is positive and 'subtracted'. |
| 0x07 | REFLEVEL | Reference level. |
| 0x08 | DISPRANGE | Display range. |
| 0x09 | DISPUNIT | Display unit. |
| 0x0A | PULSEMODE | Pulse mode – 1 is on, 0 is off. |
| 0x0B | DEMODMODE | Demodulator mode – 0 is off, 1 is AM, 2 is FM. |
| 0x0C | SPECPROC | Spectrum processing mode program number. This program is called after a sweep has completed. The default setting is program 150, which evaluates the current markers and calculates current Field Power (See disassembly of the program). |
| 0x0D | ANTTYPE | Antenna type. |
| 0x0E | CABLETYPE | Cable Type. -1: no cable 0: 1m standard cable |
| 0x1E | CENTERFREQ | Center Frequency of the sweep in MHz. The value modifies STARTFREQ and STOPFREQ with center +/- Span/2. |
| 0x1F | SPANFREQ | Modifies STOPFREQ as STARTFREQ + SPANFREQ. |
| 0x20 | USBMEAS | Measurement data to USB mode. If not 0, measurement values are sent to USB. if 0, no data is sent to USB. |
| 0x21 | USBSWPRST | Reset current sweep. If this variable is not 0, the current sweep is aborted and restarted with current values of STARTFREQ and STOPFREQ. |
| 0x30 | LOGFILEID | Log. file id. Write log file data to this file (see Program 200). |
| 0x31 | LOGSAMPCNT | Number of Samples to write to logfile. After each interval, this number is decremented and then the program 200 is called. |
| 0x32 | LOGTIMEIVL | If the sweep is finished before this interval is reached, the log event isn’t generated. |
| 0x41 | SPECDISP | Spectrum display mode. 1 shows the current value as pixel bar, 0 doesn't update the display to avoid override of bar graphs or text. |
| 0x42 | PEAKDISP | Peak display mode. 0 shows the Frequency of the markers in field 4-6 of the display and the power in Fields 10-12, 1 is visa versa. |
| 0x43 | MARKMINPK | Minimum Power to be treated as a marker. |
| 0x44 | RDOUTIDX | The display shows a vertical line at this pixel column and reports the value at this as marker. The value -1 disables this feature. |
| 0x60 | RBWFSTEP | RBW frequency in MHz, readonly. The RBW can be setup by the RESBANDW var. |
| 0x61 | ANTGAIN | Nominal antenna gain in dB. |
| 0x80 | PEAK1POW | Peak 1 power (Readonly). |
| 0x81 | PEAK2POW | Peak 2 power (Readonly). |
| 0x82 | PEAK3POW | Peak 3 power (Readonly). |
| 0x84 | PEAK1FREQ | Peak 1 freq. (Readonly). |
| 0x85 | PEAK2FREQ | Peak 2 freq. (Readonly). |
| 0x86 | PEAK3FREQ | Peak 3 freq. (Readonly). |
| 0xC0 | STDTONE | Output standard speaker tone, write the duration of the value. |

# Usb Connector – C++ Code

The files usbconnector.cpp and usbconnector.h show the operation encapsulateded as C++ Class. The class Cusbconnection has 3 major sections: Static operators to get an instance, read/write support and a background thread to read measurement results. The simplest way to start a scan on the spectran is (Sweep 1.1 GHz to 1.2 GHz with 3 MHz ResBW):

conn = Cusbconnection::getconnection(NULL);

conn -> writeenvironment(SV_STARTFREQ, 1100.0);

conn -> writeenvironment(SV_STOPFREQ, 1200.0);

conn -> writeenvironment(SV_RESBANDW, 1);

…

delete conn;

## Obtain a connection

The connection can be created by the static method getconnection(name). If the name is NULL and only one Spectran is connected, that one will be choosen. If the name is given, the device with that serial number is opened. The device name can be set by the LCS software.

Internally the dopen is called which setups the FTDxx driver and calls verifyid. The verifyid syncs with the Spectran, so it’ll accept commands.

In the current code, there will be only one connection at a time, getconnection return that one until closeall is called and thereby the old connection is deleted.

To check, wether the connection is valid, the geterrorstate function can be used. It returns the string of the error or NULL in case of no error. To check the working communication, verifyid with Parameter force can be called. This shouln’t be done very often, because it takes time.

## Read/Write commands

There are several write routines to make programming easy. The easiest way to access environment variables is readenvironment and readenvironment. Other commands can write via writebytes etc.

The best way to read the answer is readcmd. It reads complete command blocks from the device until the expected command is returned. The AMPFREQ data blocks, which may arrive at any time, are buffered, unexpected replies to other commands are dropped.

## Read measurement data

Measurement data can be read synchronous and asynchronous.

The synchronous mode:

- Write 1 to the USBMEAS variable

- Call readcmd(NULL, SV_USBMEAS, 0)

- Call readdata(target, 1)

- Repeat the last two steps until all data is arrived, then write 0 to the USBMEAS variable

The readcmd doesn’t need a buffer because all measurement data is read to the internal data buffer and transferred from there with the readdata call.

The asynchronous mode:

When data arrives, the reader thread sends a message to a window. The target window and the message id can be specified with the startreader. The third parameter of startreader is pollinterall, the thread will sleep for that time before it queries for new data.

At least after 100 ms the configured message is sent to the configured target window via a timer. The Message handler must call readdata(target, maxcount), which will return the number of values copied to the array. The timer is used to avoid one new data” trigger for each value. The stopreader stops the thread, the pausereader avoids callbacks.

# Spectran Assembler

The Spectran has a build in Stack Interpreter. The LCS Filemanager Utility contained in the LCS Software from www.elektrosmog.de can assemble and upload programs.

Programs are stored in the EEPROM of the Spectran and identified by a file number. Some special file numbers are listed in the following table:

| 100-109 | Key 0 to Key 9 |
| --- | --- |
| 110 | Key DOT |
| 111 | Key Shift |
| 120-123 | Key up, down, left, right |
| 130 | Key Clear |
| 131 | Key Enter |
| 150 | Executed after each completed Sweep, depending on display mode, the current power for the maximum marker is calculated. |
| 170 | Called by program 150. If the Spectran is in DECT-mode this program shifts the DECT-channel |

Each Assembler opcode can have multiple parameters which are retrieved from the stack. To add 5+4, first the constant 5 must be loaded on the stack, then the constant 4 and then the add opcode must be executed. The add opcode will take the last two values from the stack and replace it with the result of the operation.

Only floating point variables are used.

Labels are alphanumeric names followed by a colon.

The names T1, T2 and T3 reference the last three stack elements, where T1 is the top of stack.

The assembler has the following sections:

- Read/Write 256 memory variables

- Read/Write environment variables to control the sweep or evaluate sweep results

- Load constant values

- Jump to Labels / Call subroutines

- Math including Add, Sub, Mul, Div, Modulo, Logarithm, Power

- Output Values

## Opcode Reference

| Name | pop/ push | ## Description |
| --- | --- | --- |
| Memory/IO Access |  |  |
| LDSHCO | 0/1 | Load a constant from range –128 to 127 on the stack, needs less space than LDCONST. Parameters: the value |
| LDCONST | 0/1 | Load a float constant on the stack. Parameters: the float value or hexadecimal value |
| CHSIGN | 1/1 | Change the sign of the topmost stack variable. |
| SWAP | 2/2 | Exchange the 2 topmost stack values. Useful when operands are not in the expected order. |
| DUP | 1/2 | Duplicate the last value on the stack. |
| LOADSET | 0/1 | Load an environment variable on the stack. Parameters: the name or the index of the variable. |
| STORESET | 1/0 | Store the topmost stack value in the environment variable. Parameters: the name or the index of the variable. |
| SETUP | 0/0 | Setup multiple environment variables at once. The number of Parameters is not fixed. Syntax: Setup <name>:value <name>:value … Example: SETUP STARTFREQ:4000 STOPFREQ:5000 RESBANDW:1 VIDBANDW:1 SWEEPTIME:1000 ATTENFAC:-10 REFLEVEL:10 DISPRANGE:100 PULSEMODE:0 |
| LOADVAR | 0/1 | Load a variable from memory on the stack. Parameters: the number of the variable 0-255 |
| STOREVAR | 1/0 | Save the topmost stack variable to memory Parameters: the number of the variable 0-255 |

| ## Control opcodes |  |  |
| --- | --- | --- |
| NOP | 0/0 | Do nothing, useful in hexmode |
| JUMP | 0/0 | Jump to a label that’s close to the current location, can jump only 127 bytes, that’s mostly about 40 instructions. Parameters: The target label. |
| AJUMP | 0/0 | Jump to a label, no matter how far it is. Parameters: the name of the label |
| ACALL | 0/0 | Call a sub routine, the subroutine must be finished by RET. Parameters: The target label. |
| RET | 0/0 | Return from a subroutine. |
| STOP | 0/0 | Stop program execution. At the end of a program is an implicit STOP. |
| CHAIN | 1/0 | Execute the program with the number of the topmost stack variable. The execution doesn’t return. |
| JUMPGE | 2/0 | Jump if the T1 is greater or equal than T2. Parameters: The target label. |
| JUMPLT | 2/0 | The opposite of JUMPGE Parameters: The target label. |
| JUMPEQ | 2/0 | Jump if the T1 and T2 differ. Parameters: The target label. |
| JUMPNZ | 1/0 | Jump if T1 is different from zero. Parameters: The target label. |
| JUMPZ | 1/0 | Jump if the topmost stack variable is zero. Parameters: The target label. |

| ## Math opcodes |  |  |
| --- | --- | --- |
| ADD | 2/1 |  |
| SUB | 2/1 |  |
| MUL | 2/1 |  |
| DIV | 2/1 |  |
| POW | 1/1 | Raise power on top of stack |
| SQUARE | 1/1 | Square top of stack |
| SQRT | 1/1 | Square root top of stack |
| EXP | 1/1 | Exp. top of stack |
| LOG | 1/1 | Log. top of stack |
| LOG10 | 1/1 | Log. 10 top of stack |
| SIN | 1/1 | sin top of stack |
| COS | 1/1 | cos top of stack |
| IMOD | 2/1 | Integer modulo T2 by T1 |
| LOGNOT | 1/1 | Logical not |
| CHECKRNG | 3/1 | Check if T3 is between T2 and T1 |
| MAX | 2/1 | Build maximum of T1 and T2 |

| ## File Operations |  |  |
| --- | --- | --- |
| FILEREAD | 2/2 | Read value from file T1 at offset T2 (words, not bytes). Returns: T1 = state (1 OK, 0 No Data, -1 = Error) T2: If state is 1, the value |
| FILEWRITE | 2/1 | Write (append) value T2 to file with id T1. Returns: T1 = state (1 OK, 0 Filesystem full, -1 Error) |
| FILEDEL | 1/1 | Delete file with id T1 |

| ## Display Commands |  |  |
| --- | --- | --- |
| CLRFLD | 0/0 | Clear Field Parameters: 1. The field ID. |
| PRINTSTR | 0/0 | Print a string Parameters: 1. The field ID. 2. The string. |
| PRINTINT | 1/0 | Print Integer at T1. Parameters: 1. The field ID. |
| PPIXTEXT | 0/0 | Print Text in main pixel display Parameters: 1. Flag: Invert text. 2. The row/col of the text. The row is in the high byte of the value, the easiest way to encode row/col is to use hex values. The Row/Col are expressed in Text cells, not pixels. 3. The string. |
| PPIXDBL | 1/0 | Print a String value. The Parameters are the same as PPIXTEXT. Some string elements are interpreted special: #: print the number here. The number of #’s behind a dot interprets the precision, leading #’s count the minimum number of digits +/-: print the sign here. Using +, there will be a sign if the value is positive, using – there is onle a sign, when the value is negative, otherwise it’s a Space. .,: Dezimal dot. |
| PPICXBAR | 1/0 | Print a bar. The T1 can contain a value from 0.0 to 1.0. Parameters: 1. The text-row of the bar, no column selection. |
| ## Field ID’s |  |  |
| 0 | The main pixel field |  |
| 1 | The circle power bar at top. |  |
| 2 | The medim sized display below the power bar, 6 chars |  |
| 3 | The big size display below, 4 chars |  |
| 4-6 | The marker fields are divided in 3 Sections: a medium sized 4 digit field, a very small 3 digit field beside and a small size 6 digit field below. The ID’s 4-6 are the three 4 digit fields. |  |
| 7-9 | The ID’s 7-9 are the 3 digit fields. |  |
| 10-12 | The ID’s 10-12 are the 6 digit fields. |  |
| < 0 | If the field ID is small than 0, the sign is changed and the operation is executed but the field isn’t reset before. The field 0 isn’t reset anyway. |  |

## Example code

The described example is the current DECT scan implementation. It uses 2 variables (03 and 04). As long as var 3 is 0, program 170 is called after each sweep. If var 3 isn’t 0, program 170 is called only once directly after programm 100. Var 4 contains the current executed DECT channel. If var 3 is zero the program 170 will iterate the DECT channels.

File 100 (Key 0):

## # setup the scan environment: frequency of the first slot, 3 MHz RESBW,

## # Full VidBW, 50 ms sweep time, pulse mode active.

## SETUP

## STARTFREQ:1890 STOPFREQ:1890

## RESBANDW:1 VIDBANDW:0

## SWEEPTIME:50 ATTENFAC:-10

## REFLEVEL:0 DISPRANGE:100

## PULSEMODE:1

# load variable number 3 on the stack. This variable contains the flag: autoincrement channel or explizit choose DECT channel

## LOADVAR 03h

# load it again

## DUP

# if the var 3 containes 0 then jump to LABEL-1 (don’t write 0 to var 4, var 4 is automatically incremented by the program 170)

## JUMPZ LABEL-1

# store 0 to variable number 4

## LDSHCO 00h

## STOREVAR 04h

## LABEL-1:

# load var 4 (next executed channel) and build maximum with var 3 (its still on the stack)

## LOADVAR 04h

## MAX

#invert the result, => if var 3 is 0 and var 4 is 0 then write 1 to var 3, otherwise var 3 stays 0

## LOGNOT

## STOREVAR 03h

# now continue at program 170, which is also called after the sweep is completed and var 3 is 0

## LDCONST 170

## CHAIN

## STOP

File 170 (called after the last scan completed and var 3 containes 0, chained by program 150)

# load start and stop frequency. If stop frequency is lower than start frequency, that indicates a different program was activated, so jump to LABEL-1, where the DECT-Scan is reset

## LOADSET STARTFREQ

## LOADSET STOPFREQ

## JUMPLT LABEL-1

# load start frequency and check if it’s below 1880, that indicates a different program was activated, so jump to LABEL-1

## LOADSET STARTFREQ

## LDCONST 1880

## JUMPLT LABEL-1

# load stop frequency and check if it’s above 1900, that indicates a different program was activated, so jump to LABEL-1. The order on the stack was different, so JUMPLT compares the other direction.

## LDCONST 1900

## LOADSET STOPFREQ

## JUMPLT LABEL-1

# seems like we’re still in DECT mode. Calculate the slot Frequency by 1897.3 - Channel * 1.728.

## LDCONST 1897.3

## LOADVAR 04h

## LDCONST 1.728

## MUL

## SUB

# duplicate the last result and store it in STARTFREQ and in STOPFREQ.

## DUP

## STORESET STARTFREQ

## STORESET STOPFREQ

# load the current executed channel and display it in field 03

## LOADVAR 04h

## PRINTINT 03h

# print the string DECT- in field –3, the negative sign avoids the implizit clear field

## PRINTSTR FDh, "DECT-"

# load var 4 (current channel), add 1, modulo 10 for the next channel and store it in var 4

## LOADVAR 04h

## LDSHCO 01h

## ADD

## LDSHCO 0Ah

## IMOD

## STOREVAR 04h

# stop program here

## STOP

# jump target: store 0 in var 3 and 4. When there’s a 0 in var 3, the programm won’t be called anymore from the Scan Program

## LABEL-1:

## LDSHCO 00h

## STOREVAR 03h

## LDSHCO 00h

## STOREVAR 04h

## STOP